COMMERCIAL SUBDIVISION – FINAL PLAT COMPLETENESS REVIEW

Commercial Subdivision - Defined

Commercial subdivision includes any commercial, industrial, or multifamily residential subdivision, or any combination thereof, as more particularly defined in CMC 15.01.040.

Commercial Subdivision - Approval Procedure

Commercial Subdivisions require a three-step review process including concept plan, preliminary plat, and final plat review. Concept plans are reviewed and approved by the Planning Commission (with a public hearing). Preliminary plats are reviewed and approved by the Planning Commission (with a public hearing). Final plats are reviewed and approved by the City Council(with a public hearing).

Commercial Subdivision – Final Plat

A final plat is required for all subdivisions. It ensures that the subdivision complies with City ordinances and regulations before it is finalized and recorded with the Davis County Recorder. The final plat reflects the requirements and conditions established during the preliminary plat process. No final plat may be recorded without formal approval by the City’s authorized land use authority, for commercial subdivision this is City Council.

Commercial Subdivision – Final Plat Completeness Review

The completeness review is a preliminary step conducted by the Community Development Director to determine if the final plat application is complete and eligible for substantive review. If the application is not complete, the Community Development Director shall notify the applicant that the application is incomplete and provide information regarding deficiencies. No further action will be taken on incomplete applications. The applicant has 90 days from the date of City notice of application deficiencies to correct the deficiencies or the application will be considered withdrawn. Incomplete applications are not entitled to vested rights. Once an application is deemed complete, the Community Development Director will notify the applicant that the application is complete, and the application will be processed and reviewed for compliance with City ordinances and standards.

Commercial Subdivision – Approval by City Council and Recording

The City Council shall hold a public meeting to review proposed final plats for commercial subdivisions. During the review, the Council will ensure the plat complies with City code, State law, and all conditions of preliminary plat approval. If in compliance, the Council may approve the final plat. If not, the Council may deny the plat, with reasonings. The Council may also impose reasonable conditions or requirements to be met before final approval. Final plats have no legal effect unless signed by the Mayor, attested by the City Recorder, and recorded. Only the City Recorder may record final subdivision plats; developers are not authorized to do so. Once all required bonds (CMC 15.04.090), fees, approvals, signatures, and conditions are satisfied, the City Recorder may submit the final plat to the Davis County Recorder for recordation.

Commercial Subdivision – Expiration

Final plat approval shall expire and become void if the approved plat is not recorded within twelve (12) months of the approval date. For good cause shown, the applicant may request a one-time extension of up to six (6) months by petitioning the City Council before the expiration date. The city council must approve the extension prior to the expiration, so planning accordingly is pivotal. Extensions may be denied if deemed detrimental to the City. If subdivision-related fees have increased since the original approval, the City may require updated bond estimates and payment of any applicable fee increases or additional bond funds as a condition of granting the extension.

Commercial Subdivision – Final Plat Completeness Checklist

The following application, information, fees, and documentation is required for final plat application completeness review.

Project Name:

Date:

TRACKING HISTORY:

Application Submitted:

- Completeness Checklist DRC Meeting (1):

- Completeness Checklist DRC Meeting (2)

X –

| Application Requirement | Code Reference | Department | Comments |  |
| --- | --- | --- | --- | --- |
| Complete Online Application Form |  | Planning |  |  |
| Approved Preliminary Subdivision | 15.04.010 | Planning |  |  |
| Owner of Affidavit signed by Owner OR Application Signed by Owner’s Agent with Authorization Form | 15.04.030 | Planning/ Legal |  |  |
| Application Fees | 15.04.020 & Fee Schedule 18.040 | Planning |  |  |
| Professional Services Deposit | 15.04.025 & Fee Schedule 18.040 | Planning |  |  |
| Legal Description, meets and bounds description, of entire property |  | Engineering |  |  |
| Proposed changes to existing zoning district boundaries or zoning classifications or conditional use permits |  | Planning |  |  |
| Title Report for all property within the subdivision (current within 30 days of submittal) | 15.04.030 | Legal |  |  |
| Any agreements with adjacent property owners | 15.03.030 | Legal |  |  |
| ALTA Survey (containing information from Table A as required by the City Engineer) | 15.03.030 | Engineering/ Legal |  |  |
| The Contractors cost estimate for the project. This is needed for bonding purposes. |  | Engineering |  |  |
| Plan Set: |  |  |  |  |
| Plan Set will include the following: - Plat - Cover sheet - Existing Conditions Inventory Sheet (Survey) - Site Plan Utility Plan Sheet | ECI 12.50 Grading and Drainage Sheet, Utility Sheet, site plan 15.05 |  |  |  |
| Plans shall be drawn at a scale no smaller than one inch equals 100 feet on 24" x 36" sheets One electronic copy shall be submitted 3 paper copies shall be submitted at the time of compliance review Each Sheet should include the following: - Engineer Contact information - Name - Phone number - Firm - Address - Page number - Revision block - Date of latest draft or revision. - North arrow - Scale not smaller than 1”=100’ - Top of the drawing faces either north or west | 15.04.030 & 15.07.030(d) | Planning |  |  |
| Plat: |  |  |  |  |
| Printed on tracing linen with the outside or trim line dimensions of 19" by 30" and the border line of the plat shall be drawn in heavy lines leaving a space of at least 1.5" on the left side and at least 0.5" margin on the other sides | 15.004.030 | City Recorder |  |  |
| Markings shall be made on the tracing linen, mylar, or comparable material, with approved waterproof black ink | 15.04.030 | City Recorder |  |  |
| Proposed name of subdivision in bold | 15.04.030 | Planning |  |  |
| Signature block for: - authorized parties - property owners - lienholders | 15.04.030 | Planning/Legal |  |  |
| Blocks for signature: - Planning Commission - City Engineer - City Attorney - City Council (a signature line for the Mayor and an attestation by the City Recorder) - Davis County Recorder in the lower right corner | 15.04.030 | Planning/Legal |  |  |
| Owners dedication certificate and signature line for each property owner of record and significant lienholders | 15.04.030 | Legal |  |  |
| Notary public's acknowledgment for each signature on the plat | 15.04.030 | Legal |  |  |
| Certification of the accuracy of the plat of the subdivision and any traverse to permanent survey monuments by a land surveyor registered to practice in the State of Utah. ( Registered land surveyor's "Certificate of Survey”) | 15.04.030 | Planning/ Engineering |  |  |
| The location and dimensions to the nearest existing bench mark or monument, and section line. | 15.04.030 | Engineering |  |  |
| A correct metes and bounds description of all property included within the subdivision | 15.04.030 | Engineering |  |  |
| An accurate and complete survey to second order accuracy | 15.04.030 | Engineering |  |  |
| A traverse of the exterior boundaries of the tract, and of each block, when computed from field measurements on the ground shall close within a tolerance of one foot to 20,000 feet. | 15.04.030 | Engineering |  |  |
| The boundary lines of all lots tied to public survey monuments in the project showing: - Bearings - Distances - Dimensions - Square footage - Orientation - Topography (lines shall be slightly heavier than street and lot lines) | 15.04.030 | Engineering |  |  |
| Depiction of the survey mathematical information( bearing and distances), and data necessary to locate all monuments and to locate and retrace all interior and exterior boundary lines: - straight lines - central angle - radius and arc length of curves | 15.04.030 | Engineering |  |  |
| Depiction of the calculated Davis County coordinates for all property corners and monuments within the subdivision | 15.04.030 | Engineering |  |  |
| Lot and boundary closure calculated to the nearest 100th of a foot. | 15.04.030 | Engineering |  |  |
| Public dedicated lots, blocks, and parcels are delineated and designated with dimensions, boundaries and courses clearly shown and defined. A plat note describing the ownership and maintenance responsibilities. | 15.04.030 | Engineering |  |  |
| Parcels offered for dedication other than for streets or easements shall be clearly designated on the plat with dedication and ownership designation. A plat note describing the ownership and maintenance responsibilities. | 15.04.030 | Engineering/ Planning |  |  |
| Square footage of each lot and parcel | 15.04.030 | Planning |  |  |
| Sufficient linear, angular and curved data shall be shown to determine readily the bearing and length of the boundary lines of every block, lot and parcel which is a part thereof. No ditto marks shall be used for lot dimensions. | 15.04.030 | Engineering |  |  |
| Depicted right of way lines of each street: - Existing - Proposed | 15.04.030 | Engineering/ Public Works |  |  |
| All lots and blocks are to be numbered consecutively under a definite system approved by the Planning Commission. Numbering is consecutively throughout the subdivision with no omissions or duplications. | 15.04.030 | Planning |  |  |
| Depiction of all the side lines proposed easements of record by fine dashed lines, clearly labeled, have widths depicted, and sufficient ties thereto to definitely locate the same with respect to the subdivision as shown on the title report | 15.04.030 | Engineering |  |  |
| Depiction of all the side lines of all existing easements of record by fine dashed lines, clearly labeled, have widths depicted, and sufficient ties thereto to definitely locate the same with respect to the subdivision as shown on the title report | 15.04.030 | Engineering |  |  |
| Depiction of all stakes, monuments and other evidence indicating the boundaries of the subdivision as found on the site ( Any monument or bench mark that is disturbed or destroyed before acceptance of all improvements, shall be replaced) | 15.04.030 | Engineering |  |  |
| Depiction of all monuments placed in making the survey, including a statement as to what, if any, points were reset by ties | 15.04.030 | Engineering |  |  |
| All right of way monuments at angle points and intersections as approved by the City Engineer | 15.04.030 | Engineering |  |  |
| Cover sheet: |  |  |  |  |
| Cover Page including the following: - Engineer Contact information - Name - Phone number - Firm - Address - Page number - Development Use - Engineer Stamp - General Notes - Civil Drawlings Index - Vicinity Map | 15.05.060 | Planning |  |  |
| Site Plan will include: |  |  |  |  |
| The widths and locations of adjacent streets and other public properties within 50 feet of the subdivision via dotted line | 15.04.030 | Engineering/ Planning |  |  |
| Street in the subdivision as a continuation or an approximate continuation of an existing street, the conformity or the amount of nonconformity of such existing streets shall be accurately shown | 15.04.030 | Engineering/ Planning |  |  |
| Each lot shows the street addresses assigned thereto, and shall be according to the standard addressing system as set forth in CMC 11.01.170. | 15.04.030 | Public Works |  |  |
| Corner lots are assigned two addresses for each part of the lot having street frontage | 15.04.030 | Public Works |  |  |
| All streets within the subdivision are numbered (named streets shall also be numbered) in accordance with and in conformity with the adopted street numbering system as set forth in CMC 11.01.160. | 15.04.030 | Public Works |  |  |
| Depiction of block length measurement ( 1200 block maximum length) | 15.05.010 | Planning/ Engineering |  |  |
| In blocks over 800 feet in length, a dedicated public walkway through the block, at approximately the center of the block, may be required (walkways shall not be less than 10 feet in width) | 15.05.010 | Planning/ Engineering |  |  |
| The minimum area and dimensions of all lots shall conform to the requirements of the Zoning Code for the district in which the subdivision is located: - Depiction of all lots front, side, and rear yard setback lines from the buildable area. - Depiction of all lots frontage. - Depiction of all lots buildable area - Depiction of all lots width from the front setback. | 15.05.020 & 12.55.080 | Planning |  |  |
| Gross density for entire subdivision, if residential use is requested |  | Planning |  |  |
| Construction Plans will include the following: |  |  |  |  |
| Title block located in lower right corner of sheet to include: - Project title (subdivision, etc.) - Specific type and location of work - Name of engineer or firm preparing drawings with license number - Utah Engineers stamp shall be required on all construction plans - Date of Plans | 15.07.030(d) | Planning |  |  |
| Size of drawings shall be 24" x 36" (trim line) with 2" border on top, bottom, and right sides, and 1.5" border on the left side. | 15.07.030 | Engineer |  |  |
| Elevations referenced to U.S.G.S. datum | 15.07.030 | Engineering |  |  |
| Stationing and elevations for profiles | 15.07.030 | Engineering |  |  |
| All structures designed in accordance with minimum requirements established by the City Standards and Specifications, see below. | 15.07.030 | Public Works/ Engineering |  |  |
| Each set of plans shall be accompanied by a separate sheet of details for structures which are to be constructed | 15.07.030 | Planning/Public Works |  |  |
| Geotechnical and Soils Report (GEOTECH) | 9.06.050 &10.04.100 | Public Works |  |  |
| Utility Sheet Street/ Roadway Design Submittal Requirements: |  |  |  |  |
| Plan view for curb and gutter plans shall be shown for each side of the street | 15.07.030 | Engineering/ Public Works |  |  |
| Profiles for curb and gutter plans shall be shown for each side of the street | 15.07.030 | Engineering/ Public Works |  |  |
| Local streets shall approach an arterial or collector street at an angle of at least 85 degrees. Grades for streets shall be a minimum of 0.5% and a maximum of 12% for all streets except major and minor arterials which shall be 10% maximum. | 15.05.030 | Engineering/ Public Works |  |  |
| Cul de sacs shall: - serve no more than 20 lots or units - shall be no longer than 400 feet (measured from the center point of the turn around to the center line of the intersecting street) - terminated with a turn around of not less than 100 feet in diameter | 15.05.030 | Planning/Public Works |  |  |
| Temporary dead end street: - temporary turn-around area and recordable easement shall be provided - require improvements to be installed in temporary turn-around areas | 15.05.030 | Legal/ Public Works |  |  |
| Proof of payment for street signs to be installed by the city | 15.05.030 | Public Works |  |  |
| Permanent monuments shall be furnished, accurately established, and set by the subdivider's surveyor at such points as are necessary to definitely establish all lines of the plat except those defining individual lots | 15.05.030 | Engineering/ Public Works |  |  |
| City approved street lights shall be installed at all street intersections, school or pedestrian crossings, or at bends or curves in the street | 15.05.030 | Public Works |  |  |
| Top of curb elevations with curve data must be shown for all curb returns | 15.07.030 | Engineering/ Public Works |  |  |
| The street/roadway widths conform to the cross-section widths approved in the Preliminary Drawings and/or as required by the City Engineer. | City Standards and Details | Engineering/ Public Works |  |  |
| The street/roadway cross-section shows the placement of sidewalk, park strip, and curb & gutter as approved on the Preliminary Drawings of as required by the City Engineer. | City Standards and Details | Engineering/ Public Works |  |  |
| The pavement structure has the minimum thicknesses of asphalt and road base or as required by the City Engineer or Geotechnical/Soils Report. | City Standards and Details | Engineering/ Public Works |  |  |
| The drawings of the curb & gutter show both the plan and profile design. | City Standards and Details | Engineering/ Public Works |  |  |
| The placement of intersections conforms to the Centerville City Standards. | City Standards and Details | Engineering/ Public Works |  |  |
| The street slopes do not exceed 8% or as approved by the City Engineer. | City Standards and Details | Engineering/ Public Works |  |  |
| Public Utility Easements shown on the plats. | 15.04.030 | Engineering/ Public Works |  |  |
| The plan view drawings show lighting in the public right-of-way. | City Standards and Details | Engineering/ Public Works |  |  |
| Street signs, traffic signs, and street/roadway striping shown on plans, including proper signing and striping for school zones. | City Standards and Details | Engineering/ Public Works |  |  |
| Details for streets/roadways, pavement, sidewalk, and C&G shown on plans. | City Standards and Details | Engineering/ Public Works |  |  |
| All lots or parcels created by the subdivision shall have frontages on a dedicated street, improved to standards equal to at least 50% of minimum required width. | 15.05.020(b) | Planning/ Engineering |  |  |
| Utility Sheet Storm Darin System Submittal Requirements: |  |  |  |  |
| Depiction of storm water drainage system within the subdivision (shall be constructed of materials and according to the City Standards and Specifications and the requirements of the Storm Drainage Master Plan) | 15.05.050 | Public Works |  |  |
| Depiction of a 15 ft width right of way within the lines of any natural watercourse or channel, stream, creek, irrigation ditch, or floodplain that enters or traverses the subdivision as determined by Davis County Flood Control and/or the City Engineer | 15.05.050 | Public Works |  |  |
| Document that indicates dedicate rights of way for any pipe, conduit, channel, and retention or detention area as recommended by the City Engineer. | 15.05.050 | Public Works |  |  |
| Depiction of the storm water drainage system that is connected to an approved off site storm drain or facility acceptable to the City | 15.05.050 | Public Works |  |  |
| Depiction of all storm drain, cross gutters, dipstone inlets, and other appurtenant structures | 15.05.050 | Public Works |  |  |
| The storm drain system is of the size and type approved by the City on the Preliminary Drawings, and/or as required by the City Engineer. | City Standards and Details | Engineering |  |  |
| The storm drain system has clean-out boxes and inlet boxes placed as approved on the preliminary drawings, at all dead-end points, and as required by the City Engineer. | City Standards and Details | Engineering |  |  |
| The storm drain lines are shown on both the plan and profile drawings showing the type, depth, and slope. | City Standards and Details | Engineering |  |  |
| The storm drain plan and profile drawings have a benchmark referenced to a physical feature and a found Davis County section corner. | City Standards and Details | Engineering |  |  |
| The storm drain lines have the minimum cover as required in the specifications and the proper offset from the curb line. | City Standards and Details | Engineering |  |  |
| Double inlet storm drain boxes are placed at all invert and isolated low street areas. | City Standards and Details | Engineering |  |  |
| Bicycle-safe storm drain inlet grate covers are shown where needed. | City Standards and Details | Engineering |  |  |
| Where minimum cover questionable top elevation must be shown. | City Standards and Details | Engineering |  |  |
| Top of grate elevation and invert elevations at all: Boxes Grates Direction channels | City Standards and Details | Engineering |  |  |
| Storm Water Maintenance Plan Submitted A tentative plan by which the subdivider proposes to handle storm water drainage for an event with a 10 year return interval, as determined by the City Engineer. | 15.03.030 16.04.130 Title 16 | Public Works | Will be required at the time of plat recording. Contact Cameron Woodbury for assistance |  |
| Storm Water Maintenance Plan Agreement Signed | Title 16 | Public Works | Will be required at the time of plat recording. |  |
| Storm Water Pollution Prevention Plan (SWPPP), at time of | Title 16 16.03.040 | Public Works |  |  |
| Best Management Practices (BMPs) | 16.04.100 |  | Need at time of construction |  |
| Utility Sheet Culinary Water Requirements: |  |  |  |  |
| Size and location of culinary water lateral mains, meters, valves and hydrants | 15.07.030 | Public Works/ Engineering |  |  |
| Type of pipe | 15.07.030 | Public Works/ Engineering |  |  |
| Size and location of: - irrigation lateral mains - valves - fittings | 15.07.030 | Public Works/ Engineering |  |  |
| Size and location of: - sewer - storm drains - subdrains - manhole cleanouts | 15.07.030 | Public Works/ Engineering |  |  |
| The culinary water system design and construction are in accordance with Utah Administrative Code R309-550 Facility Design and Operation: Transmission and Distribution Pipelines. | City Standards and Details | Public Works/ Engineering |  |  |
| The culinary water system is of the size and type approved by the City on the Preliminary Drawings. | City Standards and Details | Public Works/ Engineering |  |  |
| The culinary water system has fire hydrants placed as approved on the Preliminary Drawings and at all dead-end points. Flushing hydrants are placed at all temporary dead-end points. | City Standards and Details | Public Works/ Engineering |  |  |
| Blow-offs size, type, and locations. | City Standards and Details | Public Works/ Engineering |  |  |
| The culinary water system has isolation valves installed at intersections, cul-de-sacs, and other locations required by the City Engineer. | City Standards and Details | Public Works/ Engineering |  |  |
| The culinary water system is installed at the appropriate location in the street, typically 4 feet north and east of the centerline. | City Standards and Details | Public Works/ Engineering |  |  |
| The water service line size, material, and meter sizes are shown. | City Standards and Details | Public Works/ Engineering |  |  |
| The lot numbers that are required to have a Fire Suppression System are shown, with the size and type of system shown. | City Standards and Details | Public Works/ Engineering |  |  |
| Separation of water and sewer lines maintained. | City Standards and Details | Public Works/ Engineering |  |  |
| Utility Sheet Sanitary Sewer Submittals Requirements: |  |  |  |  |
| The sanitary sewer system is of the size and type approved by the City on the Preliminary Drawings, and/or as required by the City Engineer |  | Sewer District/ Engineering |  |  |
| The sanitary sewer lines are shown on both the plan and profile drawings showing the pipe type, depth, and slope. |  | Sewer District/ Engineering |  |  |
| The sanitary sewer plan and profile drawings have a benchmark referenced to a physical feature and a found Davis County section corner. |  | Sewer District/ Engineering |  |  |
| The sanitary sewer system has manholes placed as approved on the Preliminary Drawings, at all dead-end points with lateral connections, and as required by the City Engineer. |  | Sewer District/ Engineering |  |  |
| The sanitary sewer system is installed at the appropriate location in the street, typically 9 feet south and west of the centerline. |  | Sewer District/ Engineering |  |  |
| The sanitary sewer system indicates a separate lateral from the main line to 10 feet inside the property line for each building lot. |  | Sewer District/ Engineering |  |  |
| Utility Sheet Irrigation System Submittals Requirements: |  |  |  |  |
| The pressure irrigation system is installed at an appropriate location in the street, typically in the park strip area or at the lip of the curb. |  | Public Works/ Engineering |  |  |
| A copy of the receipt for payment for secondary water service from the applicable secondary water provider must be submitted. |  | Public Works/ Engineering |  |  |
| Pipe type, depth, slope, and plan & profiles shown on plans. |  | Public Works/ Engineering |  |  |
| Boxes, manholes, locations & spacing shown on plans. |  | Public Works/ Engineering |  |  |
| Utility Sheet Power for Subdivision Submittal Requirements: |  |  |  |  |
| On-site Easements and off-site easements shown on plans. | 15.04.030 | Engineering |  |  |
| Street lights shown with proper spacing. | 15.05.030 | Public Works |  |  |
| Conflicts with any other utilities or with sight triangle addressed. | 12.55.230 | Planning |  |  |
| Compliance with power company requirements met. ( Will Serve letter) | 15.03.030 | Planning |  |  |
| Utility Sheet Fire Marshal and Public Safety Requirements: |  |  |  |  |
| Connection sizes meet fire flow requirements. | IFC 2021 10.03.120 | Fire |  |  |
| Temporary turnaround agreement/easement met. |  |  |  |  |
| Emergency access circulation/turnarounds shown |  |  |  |  |
| Parking lot turning radii requirements met. |  |  |  |  |
| Additional City Code Requirements: |  |  |  |  |
| Existing Conditions Inventory (separate sheet) | 12.50.040 |  | Before a site plan or other development approval is reviewed and approved, an existing conditions inventory, as required. Please see list below. |  |
| Analysis completed to determine whether adequate public facilities are available to service a development and whether such development will change existing levels of service or will create a demand which exceeds acceptable levels of service for: - Roadways - Intersections - Bridges - Storm drainage facilities - Water lines - Water pressure - Sewer lines - Fire and emergency response times | 12.55.050 | Engineering/ Public Works |  |  |
| Evidence that all utilities and services will be available for the subdivision and providers have been notified | 15.03.030 | Planning | - Lumin (Century Link & Quantum Fiber) - Weber Basin Water District/Deuel Creek Irrigation Co - Enbridge (Dominion Energy) - Rocky Mountain Power - South Davis Metro Fire - South Davis Sewer District - Utopia Fiber - Xfinity Cable Service\ - Davis County Health Department - UDOT Region One (1) |  |
| Explanation of RDA Request ( if applicable) | Requested- not required | Community Development Director |  |  |
| "As built" shall be provided by the developer and submitted to the City prior to final inspection. | 15.05.060 (e) |  |  |  |
| Planned Community Parks/ Common Space Requirements: |  |  |  |  |
| Parking landscaping plan shown with zero-scape requirements met. |  |  |  |  |
| Detention basin landscape requirements met. | 15.51.060 | Planning/ Engineering |  |  |
| Submit a Land Scaping Plan designed by a Utah Certified Landscape Architect | 15.05.070 | Planning |  |  |
| Submit a Parking Plan designed by a Utah Certified Transportation Engineer | 12.52.100 | Planning/ Engineering |  |  |

Additional Staff Comments for Completeness:

Comment History:

Completeness Checklist DRC Meeting (1): Please address the following items: - Review the City Details and Drawings 1701822707_Centerville City - Standard Details (2023)-adopted.pdf

Completeness Checklist DRC Meeting (1): Please address the following items:

| Title | Name | Email | Phone |
| --- | --- | --- | --- |
| Public Works Director | Mike Carlson | Mike.carlson@centervilleutah.gov | 801-677-6093 |
| Deputy Public Works Director | Dave Walker | Dave.walker@centervilleutah.gov | 801-677-6099 |
| Drainage Supervisor | Cameron Woodbury | cameron.woodbury@centervilleutah.gov | 801-677-6096 |
| Streets Supervisor | Marc Marchant | Marc.marchant@centervilleutah.gov | 801- 335- 8854 |
| Water Supervisor | Steve Hunt | Steven.hunt@centervilleutah.gov | 801-335- 8844 |
| Community Development Director | Michael Eggett | Michael.eggett@centervilleutah.gov | 801-677-6437 |
| City Planner | Sydney DeWees | Sydney.dewees@centervilleutah.gov | 801-677-6097 |
| City Attorney | Lisa Romney | Lisa.romney@centervilleutah.gov | 801-335-8842 |
| City Engineer | Kevin Campbell | Kevin.campbell@esieng.com | 801- 953-4812 |
| South Davis Fire Marshal | Cole Fessler | cfessler@sdmetrofire.gov | 801-677-2400 |
| South Davis Fire Inspector | Travis Mander | tmander@sdmetrofire.gov | 801-677-2400 |