COMMERCIAL SUBDIVISION – PRELIMINARY PLAT COMPLETENESS REVIEW

Commercial Subdivision - Defined

Commercial subdivision includes any commercial, industrial, or multifamily residential subdivision, or any combination thereof, as more particularly defined in CMC 15.01.040.

Commercial Subdivision - Approval Procedure

Commercial Subdivisions require a three-step review process including concept plat preliminary plat, and final plat review. Concept plats are reviewed and approved by the Planning Commission (with a public hearing). Preliminary plats are reviewed and approved by the Planning Commission (with a public hearing). Final plats are reviewed and approved by the City Council.

Commercial Subdivision – Preliminary Plat

A preliminary plat is required for all subdivisions. The preliminary plat minimizes changes and revisions that might otherwise occur on a final plat. It ensures compliance with the subdivision with City ordinances and regulations before the final plat is initiated.

Commercial Subdivision – Preliminary Plat Completeness Review

The completeness review is a preliminary step conducted by the Community Development Director to determine if the preliminary plat application is complete and eligible for substantive review. If the application is not complete, the Community Development Director shall notify the applicant that the application is incomplete and provide information regarding deficiencies. No further action will be taken on incomplete applications. The applicant has 90 days from the date of City notice of application deficiencies to correct the deficiencies or the application will be considered withdrawn. Incomplete applications are not entitled to vested rights. Once an application is deemed complete, the Community Development Director will notify the applicant that the application is complete, and the application will be processed and reviewed for compliance with City ordinances and standards.

Commercial Subdivision – Approval by Planning Commission

The Planning Commission shall hold a public meeting to review proposed preliminary plats for commercial subdivisions. During the review, the Commission will ensure the plat complies with City code, State law, and all conditions of conceptual plat approval. If in compliance, the Commission may approve the preliminary plat. If not, the Commission may deny the plat, with reasonings. The Commission may also impose reasonable conditions or requirements to be met before final approval.

Commercial Subdivision – Expiration

Once preliminary plat approval has been granted, the applicant may apply for final plat approval. Preliminary plat approval shall expire and have no further force or effect if a complete application for final plat approval is not submitted within twelve (12) months from the date of approval of the preliminary plat. This time period may be extended for up to six (6) months for good cause shown if the applicant petitions the Planning Commission for an extension prior to the expiration date. Only one extension may be granted for each approved preliminary plat. In those cases where a subdivision is proposed to be developed in phases, preliminary plat approval for the remaining portions of the subdivision shall not be voided if a complete application for final plat approval for the first phase is submitted within twelve (12) months from the date of approval of the preliminary plat and subsequent phases are recorded in accordance with the approved phasing plan.

Commercial Subdivision – Preliminary Plat Completeness Checklist

The following application, information, fees, and documentation is required for preliminary plat application completeness review.

Project Name:

Date:

TRACKING HISTORY:

Application Submitted:

- Completeness Checklist DRC Meeting (1):

- Completeness Checklist DRC Meeting (2)

X –

| Application Requirement | Code Reference | Department | Comments |  |
| --- | --- | --- | --- | --- |
| Complete Online Application Form |  | Planning |  |  |
| Approved Conceptual Subdivision (Conceptual Plat) | 15.03.015 | Planning |  |  |
| Owner of Affidavit signed by Owner OR Application Signed by Owner’s Agent with Authorization Form |  | Planning/ Legal |  |  |
| Application Fees | 15.03.020 & Fee Schedule 18.030 | Planning |  |  |
| Professional Services Deposit | 15.03.020 & Fee Schedule 18.040 & 18.060 | Planning |  |  |
| Legal Description of entire property | 15.03.030 | Engineering |  |  |
| Proposed changes to existing zoning district boundaries or zoning classifications or conditional use permits |  | Planning |  |  |
| Title Report for all property within the subdivision (current within 30 days of submittal) | 15.03.030 | Legal |  |  |
| Any agreements with adjacent property owners | 15.03.030 | Legal |  |  |
| ALTA Survey (containing information from Table A as required by the City Engineer) | 15.03.030 | Engineering/ Legal |  |  |
| Plan Set: |  |  |  |  |
| Plan Set will include the following: - Plat - Cover sheet - Existing Conditions Inventory Sheet (Survey) - Site Plan - Utility Plan Sheet | ECI 12.50 Grading and Drainage Sheet, Utility Sheet, site plan 15.05 |  |  |  |
| Plans shall be drawn at a scale no smaller than one inch equals 100 feet on 24" x 36" sheets One electronic copy shall be submitted 3 paper copies shall be submitted at the time of compliance review Each Sheet should include the following: - Engineer Contact information - Name - Phone number - Firm - Address - Page number - Revision block - Date of latest draft or revision. - North Arrow - Scale not smaller than 1”=100’ - Top of the drawing faces either north or west | 15.03.030 & 15.07.030(d) | Planning |  |  |
| Plat: |  |  |  |  |
| Proposed name of subdivision in bold | 15.03.030 | Planning | If a PDO, please include “PDO” at the end of the name. |  |
| Signature block for: - authorized parties - property owners - lienholders | 15.03.030 | Planning/ Legal | Visit the city Details and Drawlings |  |
| Blocks for signature: - Planning Commission - City Engineer - City Attorney - City Council (a signature line for the Mayor and an attestation by the City Recorder) - Davis County Recorder in the lower right corner | 15.04.030 | Planning |  |  |
| Owners dedication certificate and signature line for each property owner of record and significant lienholders |  | Legal |  |  |
| Notary public's acknowledgment for each signature on the plat |  | Legal |  |  |
| Certification of the accuracy of the preliminary plat of the subdivision and any traverse to permanent survey monuments by a land surveyor registered to practice in the State of Utah. ( Registered land surveyor's "Certificate of Survey”) | 15.03.030 | Engineering |  |  |
| The location and dimensions to the nearest existing bench mark or monument, and section line. | 15.03.030 | Engineering |  |  |
| Depiction of the survey mathematical information( bearing and distances), and data necessary to locate all monuments and to locate and retrace all interior and exterior boundary lines: - straight lines - central angle - radius and arc length of curves | 15.04.030 | Engineering |  |  |
| A correct metes and bounds description of all property included within the subdivision |  | Engineering |  |  |
| The boundary lines of all lots tied to public survey monuments in the project showing: - Bearings - Distances - Dimensions - Square footage - Orientation - Topography (lines shall be slightly heavier than street and lot lines) | 15.03.030 | Planning/ Engineering |  |  |
| The streets in the project showing: - Location - Widths - Distance - Dimensions - Dedication ( private or public) - Name | 15.03.030 | Planning/ Public Works/ Engineering |  |  |
| The alleys in the project showing: - Location - Widths - Distance - Dimensions - Dedication ( private or public) | 15.03.030 | Planning/ Public Works/ Engineering |  |  |
| The easements in the project showing: - Location - Widths - Distance - Dimensions - Squar Footage - Dedication ( private or public) | 15.03.030 | Planning/ Public Works/ Engineering |  |  |
| All lots shall have at least: - 10' wide front yard public utility easements - at least two other 7' wide side or rear yard public utility easements | 15.05.060(h) | Planning/ Engineering |  |  |
| The parks in the project showing: - Location - Widths - Dimensions - Squar Footage - Dedication ( private or public) | 15.03.030 | Planning/ Public Works |  |  |
| The open space in the project showing: - Location - Widths - Dimensions - Squar Footage - Dedication ( private or public) | 15.03.030 | Planning |  |  |
| Depiction of block width. The minimum width of a block shall not be less than 250 feet measured from center line of street to center line of street. | 15.05.010 | Planning |  |  |
| Depiction of block length. The maximum length of blocks shall be 1200 feet. In blocks over 800 feet in length, a dedicated public walkway through the block, at approximately the center of the block, may be required. Such walkways shall not be less than 10 feet in width unless otherwise approved by the City. | 15.05.010 | Planning |  |  |
| Cover sheet: |  |  |  |  |
| Cover Page including the following: - Engineer Contact information - Name - Phone number - Firm - Address - Page number - Development Use - Engineer Stamp - General Notes - Civil Drawlings Index - Vicinity Map | 15.05.060 | Planning |  |  |
| Site Plan will include: |  |  |  |  |
| Depiction of all lots front, side, and rear yard setback lines. | 15.05.020 | Planning |  |  |
| Depiction of all lots frontage. | 15.05.020 | Planning |  |  |
| Depiction of all lots buildable area. The buildable area is the portion of a property located outside the required setback distances from the property lines | 15.05.020 | Planning |  |  |
| Depiction of all proposed fences, retention walls, or landscaping walls greater than 4 ft | 12.55.110 | Planning/ Engineering |  |  |
| A sheet showing all retaining walls details, including their height relative to proposed grades (when involves fences, walls, grade changes, or in the Hillside Overlay Zone) | 12.21.110 | Planning |  |  |
| The subdivider shall submit a landscaping indicating how maintenance of the landscaping will be performed and by whom ( separate checklist is required) | 15.05.070 (a) | Planning |  |  |
| Subdivisions in the Hillside Overlay Zone shall comply with all landscaping provisions of the Zoning Code, including, but not limited to, provisions of CZC 12.42 | 15.05.070 (b) | Planning |  |  |
| Location, dimensions, and labeling of other features such as: - irrigation facilities - bridges - railroad tracks - buildings - bicycle racks - dumpsters - trash cans - fences/ retaining walls (with grading plan) - signage - mechanical equipment | 12.21 | Planning |  |  |
| Depiction of all off street parking | 12.52 | Planning/ Engineering |  |  |
| Depiction of snow load zones | 11.01.130 | Public Works |  |  |
| Depiction of proposed sidewalks | 10.06.020 | Public Works/ Engineering |  |  |
| Depiction of proposed signage | 12.54 | Planning/ Public Works |  |  |
| Table of Development Standards: - Setbacks from all property lines - Impervious surface sq ft and % - Square footage of building footprint - Square footage of total building floor area - Square footage of street rights of way - Residential Units - Commercial Units - Structure heights - Distance between buildings - Parking number - Density - Landscape buffering % - Floor area | 12.21.110 | Planning |  |  |
| A development schedule indicating the approximate date when construction or its stages can be expected to begin and be completed | 12.21.110 | Planning/ Engineering/ Public Works |  |  |
| Construction Plans will include the following: |  |  |  |  |
| Title block located in lower right corner of sheet to include: - Project title (subdivision, etc.) - Specific type and location of work - Name of engineer or firm preparing drawings with license number - Utah Engineers stamp shall be required on all construction plans - Date of Plans | 15.07.030 (d) | Planning |  |  |
| All drawings and/or prints shall be: - 24" x 36" (trim line) - 2" border on top, bottom, and right sides - 1.5" border on the left side | 15.07.030 (c) | Engineering | Once deemed complete, this will be requested to start compliance review |  |
| Elevations referenced to U.S.G.S. datum | 15.07.030 (d) | Engineering |  |  |
| Stationing and elevations for profiles | 15.07.030 (d) |  |  |  |
| All structures shall be designed in accordance with minimum requirements established by the City Standards and Specifications | 15.07.030 (f) | Engineering | Review the City Details and Drawings 1701822707_Centerville City - Standard Details (2023)-adopted.pdf |  |
| Each set of plans shall be accompanied by a separate sheet of details for structures which are to be constructed | 15.07.030 (e) | Engineering |  |  |
| Easement Sheet will include the following: |  |  |  |  |
| Utility easements shall be provided within the subdivision as required for public utility purposes. | 15.05.060(h) | Public Works/ Engineering |  |  |
| All utilities shall be provided through underground service, except where existing utilities are already in place. | 15.05.060 (a) | Public Works/ Engineering |  |  |
| All underground utilities shall be installed prior to the installation of road base, surfacing, curbs, gutters and sidewalks. | 15.05.060 (a) | Public Works/ Engineering |  |  |
| If underground utilities are not installed prior to street surfacing sleeves shall be required. | 15.05.060 (a) | Public Works/ Engineering |  |  |
| Utility Sheet: |  |  |  |  |
| Plans for all the street utilities shall be drawn on the same plans. All existing and proposed utilities including, but not limited to: - Sewer - culinary water - secondary water - fire hydrants - storm drains - subsurface drains - gas lines - power lines - communications lines - cable television lines - streetlights | 15.07.020 | Public Works/ Engineering |  |  |
| All streets should conform to the width designated by the Master Street Plan wherever a subdivision is in an area for which a Master Street Plan has been adopted. For territory where the Master Street Plan does not designate a street, streets shall be provided as required by the subdivision plat approving authority. | 15.05.030 (c) | Public Works/ Engineering |  |  |
| Local streets shall approach an arterial or collector street at an angle of at least 85 degrees. Grades for streets shall be a minimum of 0.5% and a maximum of 12% for all streets except major and minor arterials which shall be 10% maximum. | 15.05.030 (e) | Public Works/ Engineering |  |  |
| Dedication of half streets is prohibited unless: - a full right of way is to be obtained; curb, gutter, and sidewalk is constructed on the subdivider's side - and a minimum of 28 feet of asphalt is laid. the balance of street improvements will be provided by the applicable property owner | 15.05.030 (b) | Public Works/ Engineering |  |  |
| Cut and Fill Specifications | 15.05.030 & 15.05.070 | Public Works/ Engineering |  |  |
| Curbs, gutters, and sidewalks shall be installed on existing and proposed streets by the applicant in all subdivisions. | 15.05.030 (d) | Public Works/ Engineering |  |  |
| Both plan view and profiles for curb and gutter plans shall be shown for each side of the street; street center line profile may be eliminated. Top of curb elevations with curve data must be shown for all curb returns. | 15.07.030 (d) | Public Works/ Engineering |  |  |
| Permanent monuments shall be furnished, accurately established, and set by the subdivider's surveyor at such points as are necessary to definitely establish all lines of the plat except those defining individual lots. | 15.05.030(i) | Public Works/ Engineering |  |  |
| Protection Strips be shown to adhere to CMC 15.05.040 | 15.05.040 | Public Works/ Engineering |  |  |
| City shall furnish and install all required street signs and the cost thereof shall be charged to and paid by the subdivider in accordance with the City Fee Schedule 17.010 | 15.05.030(h) | Public Works/ Engineering |  |  |
| City approved street lights shall be depicted and installed at all: - street intersections - school or pedestrian crossings - bends or curves in the street at intervals | 15.05.060(g) & 15.05.030(j) | Public Works |  |  |
| Cul de sacs shall: - serve no more than 20 lots or units - be no longer than 400 feet, measured from the center point of the turn around to the center line of the intersecting street must be terminated with a turn around of not less than 100 feet in diameter | 15.05.030 (f) | Public Works/ Engineering |  |  |
| Downhill cul de sacs are not permitted unless adequate drainage and 100 year over flow is provided | 15.05.030 (f) | Public Works/ Engineering |  |  |
| Where a street is designed to remain only temporarily as a dead end street, a satisfactory temporary turn-around area and recordable easement shall be provided at the end thereof to remain and be available for public use so long as the dead end exists. The City may require improvements to be installed in temporary turn-around areas. | 15.05.030 (g) | Public Works/ Engineering |  |  |
| Proposed off site and on site fire hydrants. | 15.03.030 | SDFD/ Public Works |  |  |
| Hydrants located within 350 feet of any building site in the subdivision shall be charged with water and must be operable before a building permit may be issued. | 15.05.060 (c) | SDFD/ Public Works |  |  |
| Fire hydrant spacing shall not exceed 400 feet. | 15.05.060 (c) | SDFD/ Public Works |  |  |
| Fire hydrants shall be installed by the City, at the subdivider's expense, at locations determined by the City Engineer and the Fire Department. | 15.05.060 (c) | SDFD/ Public Works |  |  |
| Utility Sheet Culinary Water requirements: |  |  |  |  |
| Proposed off site and on site culinary water. | 15.03.030 | Public Works |  |  |
| Size and location of culinary water lateral mains, meters, valves and hydrants (these plans to be finalized by the City Engineer). | 15.07.030 (d) | Public Works/ Engineering |  |  |
| A culinary water supply, which must be approved by the City Engineer shall be available to each lot in the subdivision and shall be provided in conformance with the standards and rules and regulations of the City and requirements of the City Engineer. Where an approved public water supply is available, the City shall cause to be installed, at the subdivider's expense, water mains, valves, pressure reducing valves, and service laterals to each lot within the subdivision. | 15.05.060 (b) | Public Works/ Engineering |  |  |
| Utility Sheet Secondary Water/ Irrigation requirements: |  |  |  |  |
| Proposed off site and on site secondary water facilities. | 15.03.030 | Public Works |  |  |
| Secondary water for the purpose of irrigation shall be made available to each lot in all residential subdivisions | 15.05.060 (e) | Public Works/ Engineering |  |  |
| Size and location of irrigation lateral mains, valves, fittings, etc. | 15.07.030 (d) | Public Works/ Engineering |  |  |
| The applicable connection fee shall be paid to the irrigation district prior to their approval of the final plat. | 15.05.060 (e) | Public Works/ Engineering |  |  |
| The subdivider will be required, at its own expense, to construct all required off site facilities to connect to existing irrigation district facilities. | 15.05.060 (e) | Public Works/ Engineering |  |  |
| The size of service connections allowed will be determined by the irrigation district. | 15.05.060 (e) | Public Works/ Engineering |  |  |
| Construction of irrigation water facilities shall be subject to the approval of the appropriate pressure irrigation district, as signified by its letter of approval. | 15.05.060 (e) | Public Works/ Engineering |  |  |
| Utility Sheet Sanitary Sewer requirements: |  |  |  |  |
| Proposed off site and on site sanitary sewers. | 15.03.030 | Public Works |  |  |
| The subdivider shall connect with a public sanitary sewer and provide sewer mains and extend laterals from the sewer main to each lot in the subdivision | 15.05.060 (d) | Public Works/ Engineering |  |  |
| Size and location of sewer, storm drains, and subdrains, and manhole cleanouts | 15.07.030 (d) | Public Works/ Engineering |  |  |
| Type of pipe. | 15.07.030 (d) | Public Works/ Engineering |  |  |
| Utility Sheet Storm Drain requirements: |  |  |  |  |
| Proposed off site and on site storm drainage facilities | 15.03.030 | Public Works |  |  |
| The use of any subsurface drains within a subdivision may be permitted on a limited basis, as deemed appropriate by the City, in accordance with the procedures and requirements set forth in CMC 9.06 (Subsurface Water) regarding subsurface drains, and shall require videotaping prior to final inspection in accordance with CMC 9.06.080. | 15.05.060 (f) | Public Works/ Engineering |  |  |
| All storm water drainage systems, improvements and facilities installed by the developer shall comply with applicable provisions of CMC 16 (Stormwater), and shall require videotaping prior to final inspection in accordance with City Fee Schedule. | 15.05.050(e) | Public Works/ Engineering |  |  |
| The subdivider shall construct and install a storm water drainage system within the subdivision which shall be constructed of materials and according to the City Standards and Specifications and the requirements of the Storm Drainage Master Plan. | 15.05.050(a) | Public Works/ Engineering |  |  |
| The storm water drainage system for subdivider's subdivision shall be connected to an approved off site storm drain or facility acceptable to the City. | 15.05.050(c) | Public Works/ Engineering |  |  |
| The subdivider shall dedicate a right of way of 15 feet in width or greater as required by the City for storm drainage conforming substantially with the lines of any natural watercourse or channel, stream, creek, irrigation ditch, or floodplain that enters or traverses the subdivision as determined by Davis County Flood Control and/or the City Engineer. | 15.05.050(b) | Public Works/ Engineering |  |  |
| The subdivider shall dedicate rights of way for any pipe, conduit, channel, and retention or detention area as recommended by the City Engineer. | 15.05.050(b) | Public Works/ Engineering |  |  |
| Storm drain, cross gutters, dipstone inlets, and other appurtenant structures shall be provided by the subdivider (within the limits of the subdivision) as required to adequately dispose of storm waters and the 10 year frequency storm flows developed within the limits of the subdivision and the existing flows entering the proposed subdivision from adjacent properties. | 15.05.050(d) | Public Works/ Engineering |  |  |
| Approval of any Subsurface drains in accordance with applicable provisions and a copy of proposed protective covenants in all cases where subsurface drains are to be located within the subdivision, as applicable. | 9.06 | Public Works/ Engineering |  |  |
| Contour Drainage Plans | Title 16 | Public Works/ Engineering |  |  |
| Storm Water Maintenance Plan Submitted A tentative plan by which the subdivider proposes to handle storm water drainage for an event with a 10 year return interval, as determined by the City Engineer. | 15.03.030 Title 16 | Public Works/ Engineering |  |  |
| Storm Water Maintenance Plan Agreement Signed | Title 16 | Public Works/ Engineering |  |  |
| Additional City Code Requirements: |  |  |  |  |
| Existing Conditions Inventory (separate sheet) | 12.50.040 |  | Before a site plan or other development approval is reviewed and approved, an existing conditions inventory, as required. Please see list below. |  |
| Evidence that all utilities and services will be available for the subdivision and providers have been notified | 15.03.030 | Planning | - Lumin (Century Link & Quantum Fiber) - Weber Basin Water District/Deuel Creek Irrigation Co - Enbridge (Dominion Energy) - Rocky Mountain Power - South Davis Metro Fire - South Davis Sewer District - Utopia Fiber - Xfinity Cable Service\ - Davis County Health Department - UDOT Region One (1) |  |
| Submit a Land Scaping Plan designed by a Utah Certified Landscape Architect | 15.05.070 | Planning |  |  |
| Explanation of RDA Request ( if applicable) | Requested- not required | Community Development Director |  |  |
| All lots or parcels created by the subdivision shall have frontage on a dedicated street, improved to standards equal to at least 50% of its minimum required width. | 15.05.020(b) | Planning |  |  |
| "As builts" shall be provided by the developer and submitted to the City prior to final inspection. | 15.05.060 (e) |  |  |  |

Additional Staff Comments for Completeness:

Comment History:

Completeness Checklist DRC Meeting (1): Please address the following items: - Review the City Details and Drawings 1701822707_Centerville City - Standard Details (2023)-adopted.pdf

Completeness Checklist DRC Meeting (1): Please address the following items:

| Title | Name | Email | Phone |
| --- | --- | --- | --- |
| Public Works Director | Mike Carlson | Mike.carlson@centervilleutah.gov | 801-677-6093 |
| Deputy Public Works Director | Dave Walker | Dave.walker@centervilleutah.gov | 801-677-6099 |
| Drainage Supervisor | Cameron Woodbury | cameron.woodbury@centervilleutah.gov | 801-677-6096 |
| Streets Supervisor | Marc Marchant | Marc.marchant@centervilleutah.gov | 801- 335- 8854 |
| Water Supervisor | Steve Hunt | Steven.hunt@centervilleutah.gov | 801-335- 8844 |
| Community Development Director | Michael Eggett | Michael.eggett@centervilleutah.gov | 801-677-6437 |
| City Planner | Sydney DeWees | Sydney.dewees@centervilleutah.gov | 801-677-6097 |
| City Attorney | Lisa Romney | Lisa.romney@centervilleutah.gov | 801-335-8842 |
| City Engineer | Kevin Campbell | Kevin.campbell@esieng.com | 801- 953-4812 |
| South Davis Fire Marshal | Cole Fessler | cfessler@sdmetrofire.gov | 801-677-2400 |
| South Davis Fire Inspector | Travis Mander | tmander@sdmetrofire.gov | 801-677-2400 |

EXISTING CONDITIONS INVENTORY –

Existing Conditions Inventory - Defined

Existing Conditions Inventory includes plans that gather information about a development site prior to approval of a site plan or other development approval in order to avoid approving plans that create land use conflicts and to protect natural and cultural resources that may be adversely affected by construction activities.

Existing Conditions Inventory – Review Procedure

Before a site plan or other development approval is reviewed and approved, an existing conditions inventory, as required in CZC 12.50.040, shall be provided for the subject property and for the first 100 feet of adjoining property. ECI will be in conjunction with the ALTA Survey. Items in the ALTA will not be required on the ECI. If no ALTA is required, an separate ECI is required in the civil plans.

Project Name:

Date:

TRACKING HISTORY:

- Application Submitted:

- Completeness Checklist DRC Meeting (1):

- Completeness Checklist DRC Meeting (2):

X -

| Application Requirement | Code Reference | Department | Comments |  |
| --- | --- | --- | --- | --- |
| Submitted on one or more 24” x 36” sheet | 12.50.040 | Planning |  | 111 |
| Depiction of streams drainage course. | 12.50.040 | Engineering |  | 112 |
| Depiction of floodplains located on or within 100 feet of the subject property. | 12.50.040 | Engineering |  | 113 |
| Floodplains shall be as delineated on Federal Emergency Management Agency (FEMA) flood insurance rate maps | 12.50.040 | Public Works- Drainage |  | 114 |
| Floodplain elevations for a 100 year flood event shall be depicted by topographic delineation | 12.50.040 | Public Works- Drainage |  | 115 |
| Depiction of the slope categories based on the City topographic maps: - 0-10%; - 10-20%; - 20-30%; and - 30% or more. | 12.50.040 | Engineering |  | 116 |
| Depiction of sensitive soils (as identified on any soil survey prepared for a government body). | 12.50.040 | Engineering |  | 117 |
| Description of soil limitations on development | 12.50.040 | Engineering |  | 118 |
| Description of soils with severe limitation shall be noted separately | 12.50.040 | Engineering |  | 119 |
| Depiction of seasonal high water table | 12.50.040 | Engineering |  | 120 |
| Depiction of flood hazard | 12.50.040 | Public Works- Drainage/ Engineering |  | 121 |
| Depiction of poor drainage | 12.50.040 | Public Works- Drainage/ Engineering |  | 122 |
| Depiction of wetland/hydric soil conditions | 12.50.040 | Engineering |  | 123 |
| Depiction of high shrink/swell potential | 12.50.040 | Engineering |  | 124 |
| Depiction of shallow depth to bedrock | 12.50.040 | Engineering |  | 125 |
| Depiction of excessive slopes | 12.50.040 | Engineering |  | 126 |
| Depiction of high susceptibility to erosion | 12.50.040 | Engineering |  | 127 |
| Depiction of wetlands which are required to be delineated by the Clean Water Act. | 12.50.040 | Public Works- Drainage/ Engineering |  | 128 |
| Depiction wetlands by a line denotating the boundaries | 12.50.040 | Public Works- Drainage/ Engineering |  | 129 |
| A copy of any application for authorization to impact or alter wetlands | 12.50.040 | Public Works- Drainage/ Engineering |  | 130 |
| Depiction of shrub and tree stands (as indicated by recent aerial photos) | 12.50.040 | Engineering |  | 131 |
| Depiction of the shrub and tree area by a circumferential line extending to the outer perimeter of the tree or shrub canopies | 12.50.040 | Engineering |  | 132 |
| Depiction of any existing utility line, aqueduct, dam, ditch, or canal located within the area subject to the ECI. | 12.50.040 | Engineering |  | 133 |
| Depiction of any area which may be flooded in the event an aqueduct, dam, ditch, or canal fails, as contained in any study conducted and/or possessed by the City or the Utah State Engineer | 12.50.040 | Engineering |  | 134 |
| Depiction of natural features on the site and within 500 feet of any portion of it | 15.03.030 | Planning |  | 135 |
| Depiction of manmade features on the site and within 500 feet of any portion of it | 15.03.030 | Planning |  | 136 |
| Depiction of areas identified as special protection areas in this Title, the General Plan, and other documents adopted by the City, including, but not limited to, wellhead protection areas and groundwater recharge areas | 12.50.040 | Planning/ Engineering |  | 137 |
| Depiction of cultural and historic resources, including cemeteries, identified in the General Plan | 12.50.040 | Planning |  | 138 |
| Note whether the property is located within a designated historic district. | 12.50.040 | Planning |  | 139 |
| Source of the ECI data | 12.50.040 |  |  | 140 |
| If a factor is not applicable to the subject property, the ECI shall include a note to that effect. | 12.50.040 |  |  | 141 |

Additional Staff Comments:

Please address the following items: